# Sheet Metal With 2 mm thickness

## RAW MATERIAL PREPARATION

## Material Procurement

  - 2 mm thick metal sheet (pre-polished or raw)

## Initial Sheet Cutting (Blank Size)

  - Method: Water Jet Cutting

  - Why: Cold cutting, no thermal distortion, high precision for sheet metal.

  - Use: Cut the outer dimensions (base size of the sheet).

## PROFILE & HOLE CUTTING

## Outer Profile Contours

  - Method: Water Jet Cutting or Wire EDM (for tighter tolerance)

  - Use: Shapes, notches, or rounded corners.

## Hole Cutting

  - Method:

    - Wire EDM for precise holes

    - Laser Drilling or Plasma for rough holes (if allowed in "non-conventional")

  - Use: Ø6 mm, Ø5.65 mm, or any slotted holes seen in drawing.

SURFACE MODIFICATION (OPTIONAL)

- Edge Smoothing / Deburring

  - Method: Abrasive Flow Machining or Vibratory Finishing

  - Use: Smoothen edges post-cutting.

- Surface Texturing or Polishing

  - Method: Electrochemical Polishing

  - Use: For stainless steel parts to improve corrosion resistance and appearance.

# FINAL STEPS

## Cleaning

  - Method: Ultrasonic Cleaning

  - Use: Remove micro-particles, water jet residue, or polishing fluid.

## Inspection

  - Tool: CMM or Laser Scanner

  - Use: Dimensional checks for tolerances, hole position, and flatness.

SUMMARY TABLE

| Step | Method | Equipment |
| --- | --- | --- |
| Raw Sheet Cut | Water Jet | CNC Water Jet Cutter |
| Hole and Slot Machining | Wire EDM / Laser Drilling | Wire EDM / Laser Driller |
| Profile Cutting | Water Jet / EDM | CNC Water Jet or EDM |
| Edge Finishing | Abrasive Flow / Vibratory | Deburring Equipment |
| Surface Polishing | Electrochemical Polishing | Electro-polisher |
| Cleaning | Ultrasonic Cleaning | Ultrasonic Cleaner |
| Dimensional Inspection | CMM / Scanner | Coordinate Measuring Machine |

Figure 1 Base Sheet  Working Drawing

Base

## Side

Figure 2 Side Sheet Working Drawing

## Inclined 1

Figure 3 Inclined 1 Sheet Working Drawing

Figure 4 Mid Base 1 Sheet Working Drawing

Mid Base 1

Figure 5 Mid Base 2 Sheet Working Drawing

Mid Base 2

## Inclined 2

Figure 6 Inclined 2 Sheet Working Drawing

Figure 7  Top Sheet Working Drawing

Top

## Top Base

Figure 8 Top Base Sheet Working Drawing

## Triangle

Figure 9 Triangle Sheet Working Drawing

## Vertical

Figure 10 Vertical Sheet Working Drawing